# Early Ford Seals and Bearings                4/10/13

Seals:

Note NAPA dealers carry Chicago Rawhide seals per V8 Times Vol. 38 No.3 page 10

Also see Vol. 16  N0.5 page 59

35 to 48 Ford Front Hub 48-1190-A:  Victor 49187, National 471316, Chicago Rawhide CR-15730,  NAPA 15730

2.69 OD x 1.563 shaft x .375 wide

37 to 39 Ford Front Hub (One piece brake drum):  Chicago Rawhide CR-20701

28 to 38 Ford two piece rear Hub B-1175:  Victor 49024, National 5877, Chicago Rawhide CR-20702, SKF 20702

3.19” OD

38 to 48 Ford one piece rear Hub 78-1175:  Victor 49324, National 416738, Chicago Rawhide CR-20538,  NAPA 20538

2.846 OD x 2.063 (shaft) x .438 W

32 to 48 Ford Rear inner axle housing  B-4245:   National 450044 (this seal may be used for the torque tube below)

32 to 48 Ford Torque tube seal (front) B-4245:   National 450044, LYO 28004 (see previous listing above)             as measured: 1.087 shaft , 1.834 OD x 0.433 W      from V8 Times( CR 10926 1.094 Shaft x 1.828 Housing Dia. X 0.438 W)

National 450044: 1.093 shaft, 1.828 bore, 1.834 OD, 0.437 W

CR 11170 or Victor 49029

Aftermarket repair seals outside axle housing:  National 6510S,  Fel-Pro 14697,  Trostel 36320,  Chrysler T905

40 to 48 Ford Transmission side cover arms 01A-7288:  Chicago Rawhide CR-6152

37-40 Ford Transmission front bearing retainer seal 78-7052:  Victor 46410, Nat 5872S, CR 13569, Dichtomatik TCM 132032SA-BX

2.00” OD x 1.372 shaft x 0.318 thick

32-34 Steering box sector seal:  Ford used a cork gasket to replace with a modern seal the housing has to be machined.  Timken 340835, National 340835: 1.125 Shaft, 1.374 Bore, 0.125 W

35-36 steering box sector seal : CR 11209    Ford used a cork seal, sector shaft=1.122”, housing=1.875”, 0.3” width

37 to 48 Ford Steering Box Sector Seal 48-3591:  National 50151S,  Chicago Rawhide CR-11067

1.125 x 1.561 x .375

# Bearings

28-48 Spindle Bolt bearing and (28-32) Steering worm thrust   B-3123-A:  Timken T83

28-48 Front wheel inner Bearing B-1201   Timken 15118

28-48 Front wheel inner Race B-1202   Timken 15250

28-48 Front wheel outer Bearing B-1216  Timken 09074

28-48 Front wheel outer Race B-1217  Timken 09196

28-36 Rear Wheel Bearing B-1225-A   ????

36-48 Rear Wheel Bearing  68-1225-A  Bower BCA J662456 $27.65/E-20Pcs. In Stock,  Berliss 330628 $20.75/E  NAPA=Raritan A-1444

2.8125 OD x 2.0625 shaft x 0.375 Dia rollers x 1.67 wide

32-35 Generator Front Bearing (2) 18-10094  Timken A4059 bearing and A-4138 race or modify with

KYK 5202-2RSC3SRI-2 and 2 spacers see V8 Times.

37-39 Generator Front Bearing 79-10094

39-41 Generator Front Bearing  B-10094   same as transmission pilot  B-7609 see below

28-48 Pilot Bearing  B-7609  NAPA 6203-2RST, National  203FF , NCH 6203 2NSE9C3, KOYO 6203RS

Same as the 39-41 generator bearing.

28-48 Throwout Bearing 78-7580   Federal Mogul,  BCA 2065

28-48 85Hp.Trans.Countershaft Ft. and Main Shaft (short) B-7118-A   Bower J-24-12-48

32-48 85Hp.Transmission Front  B-7025-A   MRC 208SG, 1208CG, New Departure 43208

32-48 85Hp. Transmission Rear  B-7065  MRC 306SG, Federal 1306CG, Nachi 6306ZZENR two metal shields

28-48 85Hp. Trans. Countershaft Rear (long) B-7121-A  Bower J-24-12-56  $5.70/E In Stock

32-35 Steering box upper and lower worm bearings B-3571   Timken 5-BA

37-56 Steering Box upper and lower worm bearings (takes 2) 68-3571-A   Timken #5 BC

32-36 Steering Box Upper Race B-3553 replaced by Ford with 68-3553  Use as a replacement 6CE with shims for height.

37-56 Steering Box Upper Race B-3552,  Timken #6 Cup

29-36 Steering Box Lower Race B-3552  Timken #6 Cup

37-56 Steering Box Lower Race  8A-3553,  Timken #6 CE

36 Steering Box Sector Housing Bearings (2) 68-3576 Torrington NB-6

28-47 Drive Shaft front roller bearing  B-4645-A  Hyatt/Baltzer 99061

28-47 Drive Shaft front bearing split sleeve  B-4655

33-48 Ford Differential Bearings and races

32-34 Pinion race (double faced)  18-4616   Ford/Timken 18-4616, Bower 25289

35-47 Pinion race (double faced ) 48-4616   Timken 27820D

32-34 Pinion bearing (two needed) 18-4621   Timken 25132

35-47 Pinion bearing (two needed) 48-4621   Timken 27880

32-36 Carrier bearing (two needed) 18-4221   Timken 22168

37-47 Carrier bearing (two needed) 68-4221   Timken 25578

32-36 Carrier race (two needed) 18-4222   Timken 22325,  Bower 22325

37-47 Carrier race (two needed) 68-4222   Timken 25522

Columbia axle carrier bearing   Timken 368

Columbia axle race   Timken 363

32-47 Pinion pilot bearing 18-4625-A   Bower R-1304-BF